National building code

NATIONAL BUILDING CODE OF THE PHILIPPINES

| - As per BP 220, what is the minimum clear width of stairways? |  |
| --- | --- |

|  | A. | 0.60 mtrs. |
| --- | --- | --- |
|  | B. | 0.70 mtrs. |
|  | C. | 0.71 mtrs. |
|  | D. | 0,75 mtrs. |

| - What basic principle is applied in the basic planning requirements of BP 344 standards so that finding a person's way inside and outside of a building or open space shall be made easy for everyone? |  |  |
| --- | --- | --- |
|  | A. | Accessibility |
|  | B. | Usability |
|  | C. | Reachability |
|  | D. | Orientation |

| - As per BP 344, entrances with vestibules shall be provided with at least how much depth to allow a disabled person to open the door and allow his wheelchair to pass through easily? |  |
| --- | --- |

|  | A. | 1.8 mtrs. |
| --- | --- | --- |
|  | B. | 2.4 mtrs |
|  | C. | 3.0 mtrs. |
|  | D. | 3.5 mtrs |

| - As per PD 957, what is the minimum floor area of family condominium units for open market? |  |
| --- | --- |

|  | A. | 18 |
| --- | --- | --- |
|  | B. | 22 |
|  | C. | 32 |
|  | D. | 36 |

| - As per BP 220, which is not a site criteria for determining whether an economic and socialized housing shall be allowed? |  |  |
| --- | --- | --- |
|  | A. | climate condition |
|  | B. | Accessibility |
|  | C. | conformity to zoning |
|  | D. | availability of basic needs |

| - As per PD 957, what is the minimum area for a single park/playground in a residential condominium project? |  |  |
| --- | --- | --- |
|  | A. | 50 sqm. |
|  | B. | 100 sqm. |
|  | C. | 150 sqm |
|  | D. | 200 sqm. |

| - As per PD 957, what is the minimum width for roads or right -of-way for residential condominium projects? |  |
| --- | --- |

|  | A. | 6 mtrs. |
| --- | --- | --- |
|  | B. | 8 mtrs. |
|  | C. | 10 mtrs. |
|  | D. | 12 mtrs |

| - As per BP 220, which among the following is the highest in the heirarchy of basic needs of human settlements, the provision of which shall be based on the actual setting which the project is located? |  |  |
| --- | --- | --- |
|  | A. | park/playground |
|  | B. | power |
|  | C. | water |
|  | D. | movement and circulation |

| - As per BP 220, what is the minimum dimension for stairway tread and riser respectively? |  |  |
| --- | --- | --- |
|  | A. | 200 / 250 mm |
|  | B. | 250 / 200 mm |
|  | C. | 250 / 300 mm |
|  | D. | 300 / 250 mm |

| - What is the minimum floor area for open market housing units as per PD 957? |  |
| --- | --- |

|  | A. | 22 sqm |
| --- | --- | --- |
|  | B. | 30 sqm |
|  | C. | 42 sqm |
|  | D. | 54 sqm |

| - As per BP 344, a level area of not less than how many meters shall be provided at the top and bottom of any ramp? |  |  |
| --- | --- | --- |
|  | A. | 1.4 mtrs. |
|  | B. | 1.5 mtrs. |
|  | C. | 1.8 mtrs. |
|  | D. | 1.9 mtrs |

| - What is the minimum floor area for economic housing units as provided for in BP 220? |  |
| --- | --- |

|  | A. | 16 sqm |
| --- | --- | --- |
|  | B. | 18 sqm. |
|  | C. | 20 sqm |
|  | D. | 22 sqm |

| - What is the minimum lot frontage for rowhouse units in economic housing as provided for in BP 220? |  |
| --- | --- |

|  | A. | 3 mtrs. |
| --- | --- | --- |
|  | B. | 3.5 mtrs |
|  | C. | 4 mtrs. |
|  | D. | 4.5 mtrs |

| - As per BP 220, what is the minimum area for parks and playgrounds? |  |
| --- | --- |

|  | A. | 50 sqm. |
| --- | --- | --- |
|  | B. | 100 sqm |
|  | C. | 150 sqm |
|  | D. | 200 sqm. |

- As per BP 344, how much is the parking slot width for handicapped drivers and passengers providing sufficient clear space between parked cars to allow them to transfer to a wheelchair?

|  | A. | 3.6 mtrs. |
| --- | --- | --- |
|  | B. | 3.7 mtrs |
|  | C. | 3.8 mtrs. |
|  | D. | 3.9 mtrs |

| - As per BP 220, which of the following is not an important planning consideration for circulation system layout? |  |  |
| --- | --- | --- |
|  | A. | observe heirarchy of roads |
|  | B. | consider topography |
|  | C. | consider access and safety |
|  | D. | consider climate |

| - Minimum width of access road for interior or rear lots |  |
| --- | --- |

|  | A. | 3.00m |
| --- | --- | --- |
|  | B. | 4.00m |
|  | C. | 5.00m |
|  | D. | 6.00m |

| - An unoccupied space other than a yard bounded on three sides by building lines with one side bounded by another open space whether public or private. |  |  |
| --- | --- | --- |
|  | A. | Inner court |
|  | B. | Open court |
|  | C. | Through court |
|  | D. | Private court |

| - The maximum height and number of stories of every building shall be dependent upon the following: --;-- i.Character of occupancy--;-- iii. Environmental conditions--;-- ii.Type of construction --;-- iv. Capacity of public utility/service systems |  |  |
| --- | --- | --- |
|  | A. | i, ii, iii |
|  | B. | i, iii, iv |
|  | C. | i, ii, iv |
|  | D. | i, ii, iii, iv |

| - Bath and toilets shall have a minimum area of ______ with a least dimension of ______. |  |  |
| --- | --- | --- |
|  | A. | 1.2 sqm.; 0.90 mts. |
|  | B. | 1.5 sqm.; 0.75 mts. |
|  | C. | 1.5 sqm.; 0.90 mts. |
|  | D. | 1.2 sqm.; 0.75 mts. |

- Minimum ceiling height measured from the floor to the ceiling of habitable rooms provided with artificial ventilation

|  | A. | 1.8 mts. |
| --- | --- | --- |
|  | B. | 2.0 mts. |
|  | C. | 2.4 mts |
|  | D. | 2.7mts. |

| - Minimum air space for school rooms shall be _____cubic meters with ____ square meter of floor area per person |  |  |
| --- | --- | --- |
|  | A. | 3.0 ; 1.0 |
|  | B. | 4.0 ; 1.2 |
|  | C. | 3.0 ; 1.2 |
|  | D. | 4.0 ; 1.0 |

| - Which of the following statements are NOT true? – i. No building shall be constructed unless it adjoins or has direct access to a public space, yard or street on at least one of its sides--;-- ii. Where length of run for access to interior lots is 200 mts, a cul-de-sac or turn court shall be provided for each maximum run of 50 mts.--;-- iii. Parking space shall be provided inclusive of access road requirement when number of independent living units with individual entrances is increased as in multi-story apartments.--;-- iv. Only some specificbuildings shall face a public street, alley or a private road. |  |  |
| --- | --- | --- |
|  | A. | i and ii |
|  | B. | ii and iii |
|  | C. | iii and iv |
|  | D. | None of the above |

| - Rooms for human habitations shall have a minimum area of ______ with a least dimension of ______. |  |  |
| --- | --- | --- |
|  | A. | 4 sqm.; 2 mts. |
|  | B. | 6 sqm.; 2 mts. |
|  | C. | 6 sqm.; 3 mts. |
|  | D. | 9 sqm.; 3 mts |

- When the horizontal distance between the outermost portion face of the building and the inner edge of the sidewalk is more than one-half the height of the building, only a fence is required as protection during construction. Height of fence is ___.

A.1.8 mts.

B.2.0 mts.

C.2.4 mts.

D.3.00 mts

| - The minimum horizontal dimension for courts and yards. |  |
| --- | --- |

|  | A. | 1.2 mts. |
| --- | --- | --- |
|  | B. | 2.0 mts |
|  | C. | 2.4 mts |
|  | D. | 3.0 mts. |

| - Minimum air space per person for habitable rooms |  |  |
| --- | --- | --- |
|  | A. | 9 cubic mts. |
|  | B. | 12 cubic mts. |
|  | C. | 14 cubic mts |
|  | D. | 16 cubic mts |

| - An unoccupied space other than a yard bounded on two opposite sides by building lines with the other opposite side bounded by other open spaces whether public orprivate. |  |  |
| --- | --- | --- |
|  | A. | Inner court |
|  | B. | Open court |
|  | C. | Through court |
|  | D. | Private court |

| - The lowest point of elevation of the finished surface of the ground between the exterior wall of a building and a point 1.50 meters distant from the said wall. |  |  |
| --- | --- | --- |
|  | A. | Adjacent ground elevation |
|  | B. | Established ground elevation |
|  | C. | Datum |
|  | D. | Reference level |

| - The classification of use or occupancy of buildings/structures or portions thereof. |  |  |
| --- | --- | --- |
|  | A. | Character of occupancy |
|  | B. | Type of construction |
|  | C. | Zoning |
|  | D. | Land use |

| - Minimum ceiling height measured from the floor to the ceiling of habitable rooms with natural ventilation |  |
| --- | --- |

|  | A. | 2.0 mts. |
| --- | --- | --- |
|  | B. | 2.4 mts. |
|  | C. | 2.7 mts. |
|  | D. | 3.0 mts |

| - Minimum access road width for interior or rear lots on which more than 35 units of apartments, rowhouses or accesorias or group of single detached units are built |  |
| --- | --- |

|  | A. | 4.00m |
| --- | --- | --- |
|  | B. | 5.00m |
|  | C. | 6.00m |
|  | D. | 7.50m |

- Not an example of a private open space

A.Alleys

B.Yards

C.Setbacks

D.Lightwells

| - Rooms intended for any use without any artificial ventilation shall have windows with a total free area of openings equal to at least ____ percent of the floor area of the room but such opening shall not be less than 1.0 sqm. |  |
| --- | --- |

|  | A. | 5 |
| --- | --- | --- |
|  | B. | 10 |
|  | C. | 15 |
|  | D. | 20 |

| - The point of reference on the highest adjoining sidewalk or the highest adjoining ground surface. |  |  |
| --- | --- | --- |
|  | A. | Adjacent ground elevation |
|  | B. | Established ground elevation |
|  | C. | Datum |
|  | D. | Reference level |

| - Kitchens shall have a minimum area of ______ with a least dimension of ______. |  |  |
| --- | --- | --- |
|  | A. | 3 sqm.; 1 mtr. |
|  | B. | 4.5 sqm.; 1.5 mts |
|  | C. | 6 sqm.; 3 mts. |
|  | D. | 3 sqm.; 1.5 mts. |

| - Minimum air space per person for workshop, factories, and offices |  |  |
| --- | --- | --- |
|  | A. | 9 cubic mts |
|  | B. | 12 cubic mts. |
|  | C. | 14 cubic mts. |
|  | D. | 16 cubic mts |

| - An unoccupied space other than a yard bounded on all sides or around its periphery by building lines |  |
| --- | --- |

|  | A. | Inner court |
| --- | --- | --- |
|  | B. | Open court |
|  | C. | Through court |
|  | D. | Private court |

| - The vertical distance from the established grade elevation to the highest point of the opening of a flat roof, to the average height of the coping of a flat roof, to the average height of the highest gable, pitch or hip roof or to the top of the parapet, if the roof is provided with a parapet, whichever is higher. |  |  |
| --- | --- | --- |
|  | A. | Adjacent ground elevation |
|  | B. | Established grade elevation |
|  | C. | Height of buildings/structure |
|  | D. | Building bulk |

| - The classification based on the fire resistivity ratings of materials and methods of construction of buildings/structures or portions thereof. |  |  |
| --- | --- | --- |
|  | A. | Character of occupancy |
|  | B. | Type of construction |
|  | C. | Fire zone |
|  | D. | Fire hazard |

| - The floor area assigned to that use divided by the unit area per occupant. |  |  |
| --- | --- | --- |
|  | A. | density |
|  | B. | Occupant load |
|  | C. | Building bulk |
|  | D. | Floor area ratio |

| - Multi-family living units regardless of number of stories with an average living unit floor area of up to 50 sqm shall have 1 parking slot per how many living units. |  |
| --- | --- |

|  | A. | 1 |
| --- | --- | --- |
|  | B. | 4 |
|  | C. | 8 |
|  | D. | 10 |

| - Multi-family living units regardless of number of stories with an average living unit floor area of up to 50 sqm shall have 1 parking slot per how many living units. |  |
| --- | --- |

|  | A. | 1 |
| --- | --- | --- |
|  | B. | 4 |
|  | C. | 8 |
|  | D. | 10 |

| - Low-income single-detached living units in housing project areas with individual lots not more than 100 square meters shall have a pooled parking at 1 slot per how many living units. |  |
| --- | --- |

|  | A. | 5 |
| --- | --- | --- |
|  | B. | 10 |
|  | C. | 15 |
|  | D. | 20 |

| - For clubhouses, beach houses, and the like, minimum required parking is 1 slot per how many sqm of gross floor area. |  |
| --- | --- |

|  | A. | 50 |
| --- | --- | --- |
|  | B. | 75 |
|  | C. | 100 |
|  | D. | 125 |

| - For residential hotels and apartels, minimum required parking is 1 slot per how many units. |  |
| --- | --- |

|  | A. | 2 |
| --- | --- | --- |
|  | B. | 5 |
|  | C. | 8 |
|  | D. | 10 |

| - As per PD 1096, what is the minimum clear width for Group A dwelling stairs? |  |
| --- | --- |

|  | A. | 0.71m |
| --- | --- | --- |
|  | B. | 0.75m |
|  | C. | 0.80m |
|  | D. | 0.90m |

| - Which is not a requirement in applying for Building Permit? |  |  |
| --- | --- | --- |
|  | A. | Tax Declaration |
|  | B. | Latest Income Tax |
|  | C. | plans and specifications |
|  | D. | Current Real Property Tax Receipt |

| - What is referred to in works to buildings/structures involving changes in the materials used, partitioning, location/size of openings, structural parts, existing utilities and equipment but does not increase the building height and/or floor area? |  |
| --- | --- |

|  | A. | Alteration |
| --- | --- | --- |
|  | B. | Conversion |
|  | C. | Repair |
|  | D. | Renovation |

| - As per PD 1096, under what classification shall factories and workshops using less flammable or non-combustible materials be classified? |  |
| --- | --- |

|  | A. | Industrial |
| --- | --- | --- |
|  | B. | assembly |
|  | C. | business and mercantile |
|  | D. | accessory |

| - Which of the following is not covered in the framework of minimum standards and requirements for buildings under PD 1096? |  |
| --- | --- |

|  | A. | aesthetic value |
| --- | --- | --- |
|  | B. | maintenance |
|  | C. | material quality |
|  | D. | location |

- As per PD 1096, what is the size of automobile parking slot for perpendicular or diagonal parking?

A.2 x 5 mtrs.

B.2.5 x 5 mtrs.

C.2.15 x 6 mtrs

D.3 x 5 mtrs.

- As per BP 220, maintain a ratio for house and lot where ___% of the selling price for a house and lot package is allotted for the house and ____% for the lot.

A.60 % / 40%

B.70% / 30%

C.75% / 25%

D.80% / 20%

| - As per PD 1096, how many percent or parking requirements may be provided if adequate public parking lots/multi-floor parking garages are available within 200 meters of the proposed buildings/structures? |  |
| --- | --- |

|  | A. | 10 % |
| --- | --- | --- |
|  | B. | 15 % |
|  | C. | 20 % |
|  | D. | 25 % |

| - As per PD 1096, what is the minimum front setback for a Residential R-1 lot? |  |
| --- | --- |

|  | A. | 3.00 mtrs |
| --- | --- | --- |
|  | B. | 3.50 mtrs. |
|  | C. | 4.50 mtrs. |
|  | D. | 5.00 mtrs. |

| - As per PD 1096, what is the classification of buildings of masonry and wood construction with a one-hour fire-resistivity throughout and exterior walls of incombustible fire-resistive construction. |  |
| --- | --- |

|  | A. | Type I |
| --- | --- | --- |
|  | B. | Type II |
|  | C. | Type III |
|  | D. | Type IV |

| - As per PD 1096, what is the minimum air space per person for a habitable room with natural ventilation? |  |
| --- | --- |

|  | A. | 10 cum |
| --- | --- | --- |
|  | B. | 12 cum |
|  | C. | 14 cum |
|  | D. | 16 cum |

| - For residential occupancies, how many percent of an interior lot is open space? |  |
| --- | --- |

|  | A. | 25% |
| --- | --- | --- |
|  | B. | 30% |
|  | C. | 40% |
|  | D. | 50% |

| - As per PD 1096, what is the minimum width for access road to interior lots? |  |
| --- | --- |

|  | A. | 3m |
| --- | --- | --- |
|  | B. | 4m |
|  | C. | 5m |
|  | D. | 6m |

| - Which is not considered a basis of assessment for building permit fees? |  |
| --- | --- |

|  | A. | cost of construction |
| --- | --- | --- |
|  | B. | floor area |
|  | C. | lot area |
|  | D. | building use |

| - As per PD 1096, what is the minimum clear ceiling height for mezzanine floors above and below it? |  |
| --- | --- |

|  | A. | 1.70m |
| --- | --- | --- |
|  | B. | 1.80m |
|  | C. | 1.90m |
|  | D. | 2.00m |

| - As per PD 1096, what is the minimum width dimension of a court? |  |
| --- | --- |

|  | A. | 1.20m |
| --- | --- | --- |
|  | B. | 1.80 m |
|  | C. | 2.00 m |
|  | D. | 2.10 m |

| - As per PD 1096, what is the minimum clearance between the established grade of a street and/or sidewalk and the lowest under-surface of any part of a projecting balcony? |  |
| --- | --- |

|  | A. | 2.00m |
| --- | --- | --- |
|  | B. | 2.40m |
|  | C. | 2.80m |
|  | D. | 3.00m |

| - As per PD 1096, what is the maximum distance of accessible parking area for the physically handicapped from facility being served? |  |
| --- | --- |

|  | A. | 40 mtrs. |
| --- | --- | --- |
|  | B. | 50 mtrs. |
|  | C. | 60 mtrs. |
|  | D. | 70 mtrs. |

| - Failure to commence the work within a period of ______months from the date of issuance of permit is a basis for non-issuance, suspension or revocation of a Building Permit. |  |
| --- | --- |

|  | A. | 3 |
| --- | --- | --- |
|  | B. | 6 |
|  | C. | 8 |
|  | D. | 12 |

| - Which is not a ground for the non-issuance, suspension or revocation of a Building Permit? |  |  |
| --- | --- | --- |
|  | A. | non-compliance to Code |
|  | B. | inaccurate application |
|  | C. | non-submittal of logbook |
|  | D. | unauthorized change in plan |

| - As per PD 1096, what is the ratio of parking slot to gross floor area of office buildings? |  |  |
| --- | --- | --- |
|  | A. | 1 slot/100 sqm. |
|  | B. | 1 slot/125 sqm. |
|  | C. | 1 slot/ 150 sqm |
|  | D. | 1 slot/200 sqm. |

| - As per PD 1096, what is the ratio of parking slot to number of units in residentialhotels and apartels? |  |  |
| --- | --- | --- |
|  | A. | 1 slot / 5 units |
|  | B. | 1 slot / 10 units |
|  | C. | 1 slot / 15 units |
|  | D. | 1 slot / 12 units |

| - As per PD 1096, what is referred to by the total number of persons that may occupy a building or a portion thereof at any one time? |  |  |
| --- | --- | --- |
|  | A. | building occupancy |
|  | B. | occupant load |
|  | C. | building usage |
|  | D. | unit capacity |

| - As per PD 1096, what is the minimum ceiling height of the first storey of a habitable room provided with artificial ventilation? |  |  |
| --- | --- | --- |
|  | A. | 2.00m |
|  | B. | 2.10 m |
|  | C. | 2.40 m |
|  | D. | 2.70 m |

| - As per PD 1096, a wheelchair transfer area is required for every how many parking spaces in buildings/structures intended for the use or occupancy of the handicapped? |  |
| --- | --- |

|  | A. | 2 spaces |
| --- | --- | --- |
|  | B. | 3 spaces |
|  | C. | 4 spaces |
|  | D. | 5 spaces |

| - An extremely hot luminous bidge formed by passage of an electric current across a space between two conductors or terminals due to the incandescence of the conducting vapor. |  |  |
| --- | --- | --- |
|  | A. | Pyrophoric |
|  | B. | Flash Point Flash Point |
|  | C. | Electric Arc Electric Arc |
|  | D. | Fulminate Fulminate |

| - In integrated system of underground or overhead piping or both connected to a source of extinguishing agent or medium which when actuated by its automatic detecting device suppresses fire. |  |  |
| --- | --- | --- |
|  | A. | Fire protective assembly |
|  | B. | Fire alerting system |
|  | C. | Automatic fire suppression system |
|  | D. | Fire sprinkler system |

| - As per PD 1185, what is the maximum number of seats in rows of seats between aisles for places of assembly? |  |
| --- | --- |

|  | A. | 5 |
| --- | --- | --- |
|  | B. | 7 |
|  | C. | 12 |
|  | D. | 14 |

| - Passageway from one building to another or around a wall in approximately the same floor level. |  |  |
| --- | --- | --- |
|  | A. | Fire Lane |
|  | B. | Horizontal Exit |
|  | C. | Public Way |
|  | D. | Vestibule |

| - As per PD 1185, what is the maximum travel distance for places of assembly if not protected by automatic fire suppression system? |  |
| --- | --- |

|  | A. | 46 mts. |
| --- | --- | --- |
|  | B. | 56 mts. |
|  | C. | 61 mts. |
|  | D. | 71 mts |

| - A bar which extends across at least one-half of each door leaf, which will open the door if subjected to pressure. |  |  |
| --- | --- | --- |
|  | A. | Flush bolt |
|  | B. | Door jamb |
|  | C. | Door handle |
|  | D. | Panic hardware |

| - As per PD 1185, what is the minimum width for any single door in a doorway? |  |
| --- | --- |

|  | A. | 61 cm. |
| --- | --- | --- |
|  | B. | 71 cm. |
|  | C. | 81 cm. |
|  | D. | 91cm |

| - Descriptive of any material which by its nature or as a result of its reaction with other elements produces a rapid drop in temperature of the immediate surroundings |  |  |
| --- | --- | --- |
|  | A. | Oxidizing Material |
|  | B. | Cryogenic |
|  | C. | Combustible Liquid |
|  | D. | Pyrophoric (Your Answer) |

| - As per PD 1185, what is the minimum number of exits for Class A places of assembly with a capacity of more than 1,000 persons? |  |
| --- | --- |

|  | A. | 2 |
| --- | --- | --- |
|  | B. | 3 |
|  | C. | 4 |
|  | D. | 6 |

| - As per PD 1185, under what classification does fire in flammable liquids and gasses belong to? |  |
| --- | --- |

|  | A. | Class A |
| --- | --- | --- |
|  | B. | Class B |
|  | C. | Class C |
|  | D. | Class D |

| - As per PD 1185, dwelling units in row apartments shall be separated from each other by partition walls having a fire resistance rating of how many hours? |  |
| --- | --- |

|  | A. | 1 hour |
| --- | --- | --- |
|  | B. | 2 hour |
|  | C. | 3 hour |
|  | D. | 4 hour |

| - As per PD 1185, buildings over this height shall be required throughout with automatic fire sprinkler system. |  |
| --- | --- |

|  | A. | 10m |
| --- | --- | --- |
|  | B. | 12m |
|  | C. | 15m |
|  | D. | 18m |

| - As per PD 1185, which is not a factor for determining degree of hazard? |  |  |
| --- | --- | --- |
|  | A. | type of occupancy |
|  | B. | construction materials used |
|  | C. | combustibility of contents |
|  | D. | floor area |

| - As per PD 1185, between any point in an institutional sleeping room or suite and an exit access door of that room or suite shall not exceed how many meters? |  |
| --- | --- |

|  | A. | 15 mtrs. |
| --- | --- | --- |
|  | B. | 23 mtrs. |
|  | C. | 30 mtrs. |
|  | D. | 46 mtrs. |

| - Degree of hazard is determined by: i. Fire resistance rating ii. Probability that a building, structure or facility may catch fire iii. Difficulty in evacuating people iv. Difficulty in suppressing fire |  |  |  |
| --- | --- | --- | --- |
|  | A. | i, ii, iii |
|  | B. | ii, iii, iv |
|  | C. | i, iii, iv |
|  | D. | i, ii, iv |
| - A building unsafe in case of fire because it will burn easily or because it lacks adequate exits or fire escapes. |  |  |
|  | A. | Fire Abatement |
|  | B. | Fire Lane |
|  | C. | Fire Hazard |
|  | D. | Fire Trap |

| - As per PD 1185, what is the unit of exit width used to measure means of egress? |  |
| --- | --- |

|  | A. | 50 cm. |
| --- | --- | --- |
|  | B. | 55 cm. |
|  | C. | 60 cm. |
|  | D. | 65 cm. |

| - As per PD1185, what is the minimum headroom clearance for Class B stairs? |  |
| --- | --- |

|  | A. | 1.95m |
| --- | --- | --- |
|  | B. | 2.00m |
|  | C. | 2.05m |
|  | D. | 2.10m |

| - A rocket or liquid propellant which consist of combinations of fuels and oxidizers which ignite spontaneously on contact with each other. |  |  |
| --- | --- | --- |
|  | A. | Hypergolic Fuel |
|  | B. | Cryogenic |
|  | C. | Corrosive Liquid |
|  | D. | Combustible Liquid |

| - Separated from other spaces of the building or structure by construction or equipment to provide a protected way of travel to the exit discharge. |  |  |
| --- | --- | --- |
|  | A. | Fire Lane |
|  | B. | Horizontal Exit |
|  | C. | Fire Exit |
|  | D. | Vestibule |

| - As per PD 1185, every room or space in educational occupancies with a capacity of over 50 persons shall have at least how many doorways? |  |
| --- | --- |

|  | A. | 1 doorway |
| --- | --- | --- |
|  | B. | 2 doorways |
|  | C. | 3 doorways |
|  | D. | 4 doorways |

| - As per PD 1185, what is the maximum force that is applied to the releasing device of the panic hardware in the direction of exit travel that shall cause the door latch to release? |  |
| --- | --- |

|  | A. | 5 kilos |
| --- | --- | --- |
|  | B. | 6 kilos |
|  | C. | 7 kilos |
|  | D. | 8 kilos |

- As per PD 1185, what is the minimum aisle width for less than 60 seats in places of assembly?

A.0.76 mtr.

B.0.83 mtr.

C.0.91 mtr.

D.1.00 mtr

| - As per PD 1185, what is the minimum width of run exclusive of nosing or projection for Class B stairs? |  |
| --- | --- |

|  | A. | 0.23m |
| --- | --- | --- |
|  | B. | 0.24m |
|  | C. | 0.25m |
|  | D. | 0.30m |

| - As per BP 220, what is the maximum length of an alley in a residential block? |  |
| --- | --- |

|  | A. | 20 mtrs. |
| --- | --- | --- |
|  | B. | 40 mtrs. |
|  | C. | 60 mtrs |
|  | D. | 80 mtrs. |

| - As per BP 220, what is the minimum clear width of stairways? |  |
| --- | --- |

|  | A. | 0.60 mtrs. |
| --- | --- | --- |
|  | B. | 0.70 mtrs. |
|  | C. | 0.71 mtrs. |
|  | D. | 0,75 mtrs. |

- As per BP 220, maintain a ratio for house and lot where ___% of the selling price for a house and lot package is allotted for the house and ____% for the lot.

A.60 % / 40%

B.70% / 30%

C.75% / 25%

D.80% / 20%

| - What basic principle is applied in the basic planning requirements of BP 344 standards so that finding a person's way inside and outside of a building or open space shall be made easy for everyone? |  |  |
| --- | --- | --- |
|  | A. | Accessibility |
|  | B. | Usability |
|  | C. | Reachability |
|  | D. | Orientation |

| - As per BP 344, entrances with vestibules shall be provided with at least how much depth to allow a disabled person to open the door and allow his wheelchair to pass through easily? |  |
| --- | --- |

|  | A. | 1.8 mtrs. |
| --- | --- | --- |
|  | B. | 2.4 mtrs |
|  | C. | 3.0 mtrs. |
|  | D. | 3.5 mtrs |